Straw Man Product Definition    IC Junior

- Compact Steerable Line Array

- 8 x 3” full range drivers rated 20/25 watts each at 8 ohms frequency range 125Hz to 18KHz

- 8 Channels power amplification 40/50 watts rms “bench power” (sustainable for 30 seconds), 25 watts continuous rms.

- Switched Mode Power Supply 400 watts from main d.c. rail. Low power d.c. rails for Analog and DSP/uC.

- DSP provides eight channels FIR processing plus 8 x bi-quad style eq. sections and overall limiter/compressor. Single channel metering feedback.

- Microcontroller provides communications to control beam(s) and DSP control as well as system monitoring and reporting.

- Redundant Analog inputs (2), +4dBu nominal sensitivity balanced inputs.

- Compact size and weight. Targets, Height: 29” (750mm), Width: 4.2” (106mm). Depth: 4.0” (100mm), Weight: 16 lbs (7.5 Kg)

- Beamsteering from Beamware II allows one set of multiple beams (no presets)

- User eq. control – how?

- Life Safety Compliance features:

  - Dual power supply/4 channel amplifier with output switchover in the event of an amplifier or power supply failure. *

  - Dual DSP, one per amplifier/power supply with output switchover in the event of a DSP failure. *

  - Fault reporting including firmware/software failure (watch dog signaling via contact closure [relay]). * Fault contacts wired to spare pair in Cat 5 cable.

  - Self test capability for drivers and electronics. **

- Ethernet communications (non-CobraNet).

- No user controls at speaker.

- Remote power control for Power Amplifier/Power Supply

- Wink Function

- Analog connectors Phoenix, Ethernet connector RJ45, power connector IEC.

- Internal wiring of power, signal and control allows up to four module array combinations into a 32 element array.

*   New capabilities not offered in IC8 or IC8-R

** Based on current open circuit detection